1.

VHDL

--*********************************************************

-- CH01 : check compiling process

--                    designed by Pei-Chong Tang, Jan. 1999

--*********************************************************

--

entity CH01 is

port

(    XC0 : in  STD_LOGIC;

XA0 : out STD_LOGIC

);

end CH01;

architecture CH01_ARCH of CH01 is

begin

--

--*********************************************************

-- simple I/O test

--

XA1 <= XC0;

--

--*********************************************************

-- end of architechture

--

end CH01_ARCH;

UCF

##############################################

#        I/O mapping for PGA board           #

##############################################

NET XA1 LOC=P29;

NET XC0 LOC=P50;

程式說明：

XC0 : in  STD_LOGIC　　宣告匯流排C是輸入

XA0 : out STD_LOGIC　　宣告匯流排A是輸出

simple I/O test

XA1 <= XC0;           當按下XC0鍵時，XA1的燈會亮

2.

--The IEEE standard 1164 package, declares std_logic, rising_edge(), etc.

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_arith.all;

use IEEE.std_logic_unsigned.all;

--This package is a version of the Synopsys package and has been

-- optimized for use with the Express compiler.

library SYNOPSYS;

use SYNOPSYS.attributes.all;

--

--*********************************************************

-- CH02 : practise VECTOR signals

--                    designed by Pei-Chong Tang, Jan. 1999

--*********************************************************

--

entity CH02 is

port

(    XA : out STD_LOGIC_VECTOR (0 to 3);

XB : out STD_LOGIC_VECTOR (0 to 7);

XC : in  STD_LOGIC_VECTOR (3 downto 0)

);

end CH02;

architecture CH02_ARCH of CH02 is

begin

--

--*********************************************************

-- VECTOR process

XA <= XC;

XB(0 downto 3) <= XC;

XB(4 downto 7) <= XC;

--*********************************************************

-- end of architechture

--

end CH02_ARCH;

UCF

##############################################

#        I/O mapping for PGA board           #

##############################################

NET XA<0> LOC = P28;

NET XA<1> LOC = P29;

NET XA<2> LOC = P35;

NET XA<3> LOC = P38;

NET XB<0> LOC = P39;

NET XB<1> LOC = P40;

NET XB<2> LOC = P44;

NET XB<3> LOC = P45;

NET XB<4> LOC = P46;

NET XB<5> LOC = P47;

NET XB<6> LOC = P48;

NET XB<7> LOC = P49;

NET XC<0> LOC = P50;

NET XC<1> LOC = P51;

NET XC<2> LOC = P56;

NET XC<3> LOC = P57;

程式說明：

XA : out STD_LOGIC_VECTOR (0 to 3);　　宣告接腳位址A0到A3為輸出

XB : out STD_LOGIC_VECTOR (0 to 7);     宣告接腳位址B0到B7為輸出

XC : in  STD_LOGIC_VECTOR (3 downto 0)  宣告接腳位址C3到C0為輸入

XA <= XC;                 當按下C0到C3對應輸出A0到A3的燈會亮

XB(0 downto 3) <= XC;     當按下 C0到C3對應輸出B0到B3的燈會亮

XB(4 downto 7) <= XC;     當按下 C0到C3對應輸出B4到B7的燈會亮

3.

程式：

The IEEE standard 1164 package, declares std_logic, rising_edge(), etc.

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_arith.all;

use IEEE.std_logic_unsigned.all;

--This package is a version of the Synopsys package and has been

-- optimized for use with the Express compiler.

library SYNOPSYS;

use SYNOPSYS.attributes.all;

--

--*********************************************************

-- CH03 : practise BIT operators

--                    designed by Pei-Chong Tang, Jan. 1999

--*********************************************************

--

entity CH03 is

port

(    XB : out STD_LOGIC_VECTOR (7 downto 0);

XC : in  STD_LOGIC_VECTOR (2 downto 0)

);

end CH03;

architecture CH03_ARCH of CH03 is

begin

--

--*********************************************************

-- 3-to-8 address decoding

--

XB(0) <= not(   not XC(2) and not XC(1) and not XC(0)); --000

XB(1) <= not(   not XC(2) and not XC(1) and     XC(0)); --001

XB(2) <= not(   not XC(2) and     XC(1) and not XC(0)); --010

XB(3) <= not(   not XC(2) and     XC(1) and     XC(0)); --011

XB(4) <= not(       XC(2) and not XC(1) and not XC(0)); --100

XB(5) <= not(       XC(2) and not XC(1) and     XC(0)); --101

XB(6) <= not(       XC(2) and     XC(1) and not XC(0)); --110

XB(7) <= not(       XC(2) and     XC(1) and     XC(0)); --111

--

--*********************************************************

-- end of architechture

--

end CH03_ARCH;

UCF

##############################################

#        I/O mapping for PGA board           #

##############################################

NET XB<0> LOC = P39;

NET XB<1> LOC = P40;

NET XB<2> LOC = P44;

NET XB<3> LOC = P45;

NET XB<4> LOC = P46;

NET XB<5> LOC = P47;

NET XB<6> LOC = P48;

NET XB<7> LOC = P49;

NET XC<0> LOC = P50;

NET XC<1> LOC = P51;

NET XC<2> LOC = P56;

#NET XC<3> LOC = P57;#

程式說明：

程式

1.XB : out STD_LOGIC_VECTOR (7 downto 0);

2.XC : in  STD_LOGIC_VECTOR (2 downto 0)

3.XB(0) <= not(   not XC(2) and not XC(1) and not XC(0)); --000

4.XB(1) <= not(   not XC(2) and not XC(1) and     XC(0)); --001

5.XB(2) <= not(   not XC(2) and     XC(1) and not XC(0)); --010

6.XB(3) <= not(   not XC(2) and     XC(1) and     XC(0)); --011

7.XB(4) <= not(       XC(2) and not XC(1) and not XC(0)); --100

8.XB(5) <= not(       XC(2) and not XC(1) and     XC(0)); --101

9.XB(6) <= not(       XC(2) and     XC(1) and not XC(0)); --110

10.XB(7) <= not(       XC(2) and     XC(1) and     XC(0)); --111

說明：由於輸出為0時LED燈才會亮，在此我們選擇0作用的輸出方式

1.宣告接腳位址B7到B0為輸出

2.宣告接腳位址C2到C0為輸入

3.到10.

| 輸入訊號 |  |  | 輸出訊號 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XC2 | XC1 | XC0 | XB7 | XB6 | XB5 | XB4 | XB3 | XB2 | XB1 | XB0 |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

電路圖：

4.

程式：

--The IEEE standard 1164 package, declares std_logic, rising_edge(), etc.

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_arith.all;

use IEEE.std_logic_unsigned.all;

--This package is a version of the Synopsys package and has been

-- optimized for use with the Express compiler.

library SYNOPSYS;

use SYNOPSYS.attributes.all;

--

--*********************************************************

-- CH04 : practise VECTOR operators

--                    designed by Pei-Chong Tang, Jan. 1999

--*********************************************************

--

entity CH04 is

port

(    XB : out STD_LOGIC_VECTOR (7 downto 0);

XC : in  STD_LOGIC_VECTOR (2 downto 0)

);

end CH04;

architecture CH04_ARCH of CH04 is

begin

--

--*********************************************************

-- 3-to-8 address decoding

--

XB <= "01111110" when XC="000" else

"10111101" when XC="001" else

"11011011" when XC="010" else

"11100111" when XC="011" else

"10101010" when XC="100" else

"11001100" when XC="101" else

"11000011" when XC="110" else

"11111111";

--*********************************************************

-- end of architechture

--*********************************************************

end CH04_ARCH;

UCF：

##############################################

#        I/O mapping for PGA board           #

##############################################

NET XB<7> LOC = P39;

NET XB<6> LOC = P40;

NET XB<5> LOC = P44;

NET XB<4> LOC = P45;

NET XB<3> LOC = P46;

NET XB<2> LOC = P47;

NET XB<1> LOC = P48;

NET XB<0> LOC = P49;

NET XC<0> LOC = P50;

NET XC<1> LOC = P51;

NET XC<2> LOC = P56;

#NET XC<3> LOC = P57;#

程式說明：

程式：

1.XB : out STD_LOGIC_VECTOR (7 downto 0);

2.XC : in  STD_LOGIC_VECTOR (2 downto 0

3. XB <= "01111110" when XC="000" else

"10111101" when XC="001" else

"11011011" when XC="010" else

"11100111" when XC="011" else

"10101010" when XC="100" else

"11001100" when XC="101" else

"11000011" when XC="110" else

"11111111";

說明：由於輸出為0時LED燈才會亮，在此我們選擇0作用的輸出方式

1.宣告接腳位址B7到B0為輸出

2.宣告接腳位址C2到C0為輸入

3.程式如表

| 輸入訊號 |  |  | 輸出訊號 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XC2 | XC1 | XC0 | XB7 | XB6 | XB5 | XB4 | XB3 | XB2 | XB1 | XB0 |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

電路圖：

5.

--The IEEE standard 1164 package, declares std_logic, rising_edge(), etc.

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_arith.all;

use IEEE.std_logic_unsigned.all;

--This package is a version of the Synopsys package and has been

-- optimized for use with the Express compiler.

library SYNOPSYS;

use SYNOPSYS.attributes.all;

--

--*********************************************************

-- CH05 : check 2-BIT MATH operator

--                    designed by Pei-Chong Tang, Jan. 1999

--*********************************************************

--

entity CH05 is

port

(    XA : out UNSIGNED (3 downto 0);

XB : out UNSIGNED (7 downto 0);

XC : in  UNSIGNED (3 downto 0)

);

end CH05;

architecture CH05_ARCH of CH05 is

signal A,B : UNSIGNED (1 downto 0);  --internal signals

begin

--

--*********************************************************

-- 2-BIT MATH operator

--

A <= XC(3 downto 2);

B <= XC(1 downto 0);

XB(1 downto 0) <= A + B - 2;         --test (A+B)

XB(3 downto 2) <= A - B + 2;         --test (A-B)

XB(7 downto 4) <= A * B;             --test (A*B)

XA(0) <= '0' when A>=B else          --test (A>=B)

'1';

XA(1) <= '0' when A>B  else          --test (A>B)

'1';

XA(2) <= '0' when A<B  else          --test (A<B)

'1';

XA(3) <= '0' when A=B  else          --test (A=B)

'1';

--

--*********************************************************

-- end of architechture

--

end CH05_ARCH;

UCF：

##############################################

#        I/O mapping for PGA board           #

##############################################

NET XA<0> LOC = P28;

NET XA<1> LOC = P29;

NET XA<2> LOC = P35;

NET XA<3> LOC = P38;

NET XB<0> LOC = P39;

NET XB<1> LOC = P40;

NET XB<2> LOC = P44;

NET XB<3> LOC = P45;

NET XB<4> LOC = P46;

NET XB<5> LOC = P47;

NET XB<6> LOC = P48;

NET XB<7> LOC = P49;

NET XC<0> LOC = P50;

NET XC<1> LOC = P51;

NET XC<2> LOC = P56;

NET XC<3> LOC = P57;

程式說明：

此程式為使用二進制來運算，其範圍大約為0到3的加減(2位元)，且由於輸出為0時LED燈才會亮，在此我們選擇0作用的輸出方式。

程式：

1.XA : out UNSIGNED (3 downto 0);

2.XB : out UNSIGNED (7 downto 0);

3.XC : in  UNSIGNED (3 downto 0)

4. A <= XC(3 downto 2);

5. B <= XC(1 downto 0);

6.XB(1 downto 0) <= A + B - 2;

7.XB(3 downto 2) <= A - B + 2;

8.XB(7 downto 4) <= A * B;

9.XA(0) <= '0' when A>=B else

'1';

10.XA(1) <= '0' when A>B  else

'1';

11.XA(2) <= '0' when A<B  else

'1';

12.XA(3) <= '0' when A=B  else

'1';

程式說明：

1.宣告接腳位址A3到A0為輸出

2.宣告接腳位址B7到B0為輸出

3.宣告接腳位址C3到C0為輸入

4.宣告C3到C2的輸入視為A

5.宣告C1到C0的輸入視為B

6.B1到B0顯示A+B-2的結果

7.B3到B2顯示A-B-2的結果

8.B7到B4顯示A*B結果

9.當A>=B時A0的燈會亮

10.當A>B時A1的燈會亮

11.當A<B時A2的燈會亮

12.當A=>B時A3的燈會亮

電路圖：

7.

程式：

VHDL：

--The IEEE standard 1164 package, declares std_logic, rising_edge(), etc.

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_arith.all;

use IEEE.std_logic_unsigned.all;

--This package is a version of the Synopsys package and has been

-- optimized for use with the Express compiler.

library SYNOPSYS;

use SYNOPSYS.attributes.all;

--

--*********************************************************

-- CH07 : practise synchronize UP/DOWN counter

--                    designed by Pei-Chong Tang, Jan. 1999

--*********************************************************

--

entity CH07 is

port

(    CLK : in  STD_LOGIC;

XB  : out STD_LOGIC_VECTOR (7 downto 0);

XC  : in  STD_LOGIC_VECTOR (3 downto 0)

);

end CH07;

architecture CH07_ARCH of CH07 is

signal SAMPLE,FLT,DIF : STD_LOGIC;

begin

--

--*********************************************************

--

-- 15-BIT free counter

--

FREE_COUNTER : block

signal Q   : STD_LOGIC_VECTOR (14 downto 0);

signal DLY : STD_LOGIC;

begin

process (CLK)

begin

if CLK'event and CLK='1' then    --CLK rising

DLY <= Q(14);               --delay

Q   <= Q+1;                 --UP counter

end if;

end process;

SAMPLE <= Q(14) and not DLY;          --differential

end block FREE_COUNTER;

--

--*********************************************************

--

-- keyboard debouncing

--

DEBOUNCING : block

signal D0,D1 : STD_LOGIC;

begin

process (CLK)

begin

if CLK'event and CLK='1' then    --CLK rising

if SAMPLE='1' then          --sampling control

D1<=D0; D0<=not XC(0); --delay

FLT <= ((D0 and D1) or FLT) and (D0 or D1); --RS-F/F

end if;

end if;

end process;

end block DEBOUNCING;

--

--*********************************************************

--

-- differential signal

--

DIFFERENTIAL : block

signal D0,D1 : STD_LOGIC;

begin

process (CLK)

begin

if CLK'event and CLK='1' then    --CLK rising

D1<=D0; D0<= XC(0);            --delay

end if;

end process;

DIF <= D0 and not D1;                 --differential

end block DIFFERENTIAL;

--

--*********************************************************

--

-- 8-BIT synchronize up/down counter

--

UPDOWN_COUNTER : block

signal Q : STD_LOGIC_VECTOR (7 downto 0);

signal RST,EC,DIR : STD_LOGIC;

begin

process (CLK)

begin

if RST='1' then                  --counter reset

Q <= "00000000";            --Q=0

elsif CLK'event and CLK='1' then --CLK rising

if EC='1' then              --CLK enable

if DIR='0' then        --UP/DOWN direction

Q <= Q+1;         --UP   counter

else

Q <= Q-1;         --DOWN counter

end if;

end if;

end if;

end process;

RST <= not XC(3);                     --RST signal

EC  <= DIF and XC(2);                 --EC  signal

DIR <= not XC(1);                     --DIR signal

XB  <= not Q;                         --output signal

end block UPDOWN_COUNTER;

--

--*********************************************************

-- end of architechture

--

end CH07_ARCH;

UCF：

##############################################

#        I/O mapping for PGA board           #

##############################################

NET CLK   LOC = P13;

NET XB<0> LOC = P39;

NET XB<1> LOC = P40;

NET XB<2> LOC = P44;

NET XB<3> LOC = P45;

NET XB<4> LOC = P46;

NET XB<5> LOC = P47;

NET XB<6> LOC = P48;

NET XB<7> LOC = P49;

NET XC<0> LOC = P50;

NET XC<1> LOC = P51;

NET XC<2> LOC = P56;

NET XC<3> LOC = P57;

程式說明：

程式：

if CLK'event and CLK='1' then    --CLK rising

D1<=D0; D0<= XC(0);            --delay

end if;

說明：在此，我們只更改鍵盤的彈跳DELAY時間，所以在差分電路(DIFFERENTIAL SIGNAL)這邊，將本來輸入的FLT訊號改成原本輸入的XC(0)，這樣，彈跳電路的功能將喪失，並且，我們會發現輸出的結果，將不在想原來般完美，而是變成按一下可能有好幾個輸入，以致輸出會不規則。

8.

實驗步驟：

電路合成(在軟體介面按下Synthesis)會出現一個視窗(以下為執行步驟)選擇主程式(Top Level&Version Name)選用FPGA元件(Family)

選擇FPGA包裝(Device)選擇FPGA速度(Speed grade)

選擇編譯速限(Target Clock Freguency)執行(Run)

電路排版(Implementation)

假如過程都沒問題，就不用按任何按鍵。

FPGA程式載入因為所選的模組有三顆FPGA，本實驗只使用一顆，故設定如下:Open LoadPGA設定PGA#1對映的rbt 檔(C:\VHDLbook\CH03\ch03.rbt)

設定PGA#2對映的rbt 檔(因無使用到，故設定為ionull.rbt)

設定PGA#3對映的rbt 檔(因無使用到，故設定為ionull.rbt)

Open(載入)及可完成欲所要的功能。

程式：

ch08.vhd

------------------------------------------------------------------------------------------------------

--The IEEE standard 1164 package, declares std_logic, rising_edge(), etc.

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_arith.all;

use IEEE.std_logic_unsigned.all;

--This package is a version of the Synopsys package and has been

-- optimized for use with the Express compiler.

library SYNOPSYS;

use SYNOPSYS.attributes.all;

--

--*********************************************************

-- CH08 : practise TIMER and FREQUENCY GENERATOR

--                    designed by Pei-Chong Tang, Jan. 1999

--*********************************************************

--

entity CH08 is

port

(    CLK : in  STD_LOGIC;

XA  : out STD_LOGIC_VECTOR (3 downto 0);

XB  : out STD_LOGIC_VECTOR (7 downto 0);

XC  : in  STD_LOGIC_VECTOR (3 downto 0)

);

end CH08;

architecture CH08_ARCH of CH08 is

signal SAMPLE    : STD_LOGIC;

signal FOUT,DOUT : STD_LOGIC;

signal VALUE     : STD_LOGIC_VECTOR (7 downto 0);

begin

--

--*********************************************************

--

-- 18-BIT free counter

--

FREE_COUNTER : block

signal Q   : STD_LOGIC_VECTOR (17 downto 0);

signal DLY : STD_LOGIC;

begin

process (CLK)

begin

if CLK'event and CLK='1' then    --CLK rising

DLY <= Q(17);               --delay

Q   <= Q+1;                 --UP counter

end if;

end process;

SAMPLE <= Q(17) and not DLY;          --differential

end block FREE_COUNTER;

--

--*********************************************************

--

-- 8-BIT UP/DOWN parameter setting

--

UPDOWN_COUNTER : block

signal Q : STD_LOGIC_VECTOR (7 downto 0);

signal RST,EC,DIR : STD_LOGIC;

begin

process (CLK)

begin

if RST='1' then                  --counter reset

Q <= "00000000";            --Q=0

elsif CLK'event and CLK='1' then --CLK rising

if EC='1' then              --CLK enable

if DIR='0' then        --UP/DOWN direction

Q <= Q+1;         --UP   counter

else

Q <= Q-1;         --DOWN counter

end if;

end if;

end if;

end process;

RST <= not XC(3);                       --RST signal

DIR <= not XC(1);                       --DIR signal

EC  <= SAMPLE and not XC(0);            --EC  signal

XB  <= not Q;                           --output signal

VALUE <=   Q;                           --parameter value

end block UPDOWN_COUNTER;

--

--*********************************************************

-- programmable frequency generation

--

FREQ_GENERATION : block

signal Q      : STD_LOGIC_VECTOR (8 downto 0);

signal DLY,EC : STD_LOGIC;

begin

process (CLK)

begin

if CLK'event and CLK='1' then      --CLK rising

DLY <= Q(8);

if EC='1' then                --CLK enable

Q <= Q + ('0' & VALUE);  --integration

end if;

end if;

end process;

EC   <= SAMPLE;

DOUT <= Q(8) xor DLY;                   --differential

FOUT <= Q(8);

end block FREQ_GENERATION;

--

--*********************************************************

-- A/B phase generation

--

PHASE_GENERATION : block

signal EC,DIR,A,B : STD_LOGIC;

begin

process (CLK)

begin

if CLK'event and CLK='1' then    --CLK rising

if EC='1' then              --CLK enable

A <= DIR xor     B;    --A phase generation

B <= DIR xor not A;    --B phase generation

end if;

end if;

end process;

EC  <= DOUT;                          --base frequency

DIR <= XC(2);                         --direction control

XA(0) <= A;                           --A-phase output

XA(1) <= B;                           --B-phase output

XA(2) <= not DOUT;                    --pulse  output

XA(3) <=     FOUT;                    --square output

end block PHASE_GENERATION;

--

--*********************************************************

-- end of architechture

--

end CH08_ARCH;

程式說明：

此實驗的主要功能，是一個多種波形輸出的可程式頻率產生器，其中的頻率值可藉由四個按鍵改變。整個電路分成4個模組，分別為自由計數器、上下計數器、頻率控制器及波形產生器。我們利用18位元的自由計數器產生16Hz的抽樣訊號，並利用微分電路縮小至一個CLK週期，提供給正反計數器及頻率產生器使用。而我們在利用正反計數器做遞加遞減工作。接下去就是一個可程式的頻率產生器，利用積分除頻器產生一個震盪輸出，而產生的頻率為16/256*SAMPLE。最後利用波形產生電路將前面之頻率換成AB雙相形式輸出。

輸出結果：

輸入按鍵XC(0)~XC(3)分別控制

XC(0)：計數輸入，每次按鍵控制計數器連續加一或減一。

XC(1)：正反控制，當按下時選擇下數，放開時則選擇上數。

XC(2)：相位控制，當按下時輸出波形相位變反向。

XC(3)：計數歸零，當按下時計數值歸零。

在顯示方面，XB(7~0)用來顯示計時器的結果，也就是頻率的設定值，而由XA(3~0)顯示頻率控制的結果，其中

XA(0)：A相輸出波形。

XA(1)：B相輸出波形。

XA(2)：脈波輸出波形。

XA(3)：方波輸出波形。

程式：

ch08.ucf

程式說明：

##############################################

#        I/O mapping for PGA board           #

##############################################

NET CLK   LOC = P13;

NET XA<0> LOC = P28;

NET XA<1> LOC = P29;

NET XA<2> LOC = P35;

NET XA<3> LOC = P38;

NET XB<0> LOC = P39;

NET XB<1> LOC = P40;

NET XB<2> LOC = P44;

NET XB<3> LOC = P45;

NET XB<4> LOC = P46;

NET XB<5> LOC = P47;

NET XB<6> LOC = P48;

NET XB<7> LOC = P49;

NET XC<0> LOC = P50;

NET XC<1> LOC = P51;

NET XC<2> LOC = P56;

NET XC<3> LOC = P57;

程式說明：

定義其內部程式與硬體相對應之接腳。

9.

程式與結果：

VHDL：

--The IEEE standard 1164 package, declares std_logic, rising_edge(), etc.

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_arith.all;

use IEEE.std_logic_unsigned.all;

--This package is a version of the Synopsys package and has been

-- optimized for use with the Express compiler.

library SYNOPSYS;

use SYNOPSYS.attributes.all;

--

--*********************************************************

-- CH11 : digital clock with 7-segment LED control

--                    designed by Pei-Chong Tang, Jan. 1999

--*********************************************************

--

entity CH11A is

port

(    CLK : in  STD_LOGIC;                      --4MHz clock

XA  : out STD_LOGIC_VECTOR (3 downto 0);  --output port

XB  : out STD_LOGIC_VECTOR (7 downto 0);  --input/output

YC  : in  STD_LOGIC_VECTOR (3 downto 0)   --input  port

);

end CH11A;

architecture CH11A_ARCH of CH11A is

signal DB  : STD_LOGIC_VECTOR (3 downto 0);

signal SEG : STD_LOGIC_VECTOR (6 downto 0);

signal ENB : STD_LOGIC_VECTOR (3 downto 0);

begin

--

--*********************************************************

--

-- system connection for the experiment

--

SYSTEM_CONNECT : block

begin

XA    <= not ENB;

XB(7) <= not YC(0) or not YC(1) or not YC(2) or not YC(3);

XB(6 downto 0) <= SEG;

end block SYSTEM_CONNECT;

--

--*********************************************************

-- 24-bit free counter

--

FREE_COUNTER : block

signal Q : STD_LOGIC_VECTOR (23 downto 0);

signal D : STD_LOGIC_VECTOR (1  downto 0);

begin

process (CLK)

begin

if CLK'event and CLK='1' then

Q   <= Q+1;

end if;

end process;

DB <= Q(23 downto 20);

D  <= Q(22 downto 21);

ENB <= "0001" when D=0 else

"0010" when D=1 else

"0100" when D=2 else

"1000" when D=3 else

"0000";

end block FREE_COUNTER;

--

--*********************************************************

-- seven-segment LED transformation

--

SEVEN_SEGMENT : block

begin

--GFEDCBA

SEG <= "0111111" when DB=0  else

"0000110" when DB=1  else

"1011011" when DB=2  else

"1001111" when DB=3  else

"1100110" when DB=4  else

"1101101" when DB=5  else

"1111101" when DB=6  else

"0000111" when DB=7  else

"1111111" when DB=8  else

"1101111" when DB=9  else

"1110111" when DB=10 else

"1111100" when DB=11 else

"0111001" when DB=12 else

"1011110" when DB=13 else

"1111001" when DB=14 else

"1110001" when DB=15 else

"0000000";

end block SEVEN_SEGMENT;

--

--*********************************************************

-- end of architechture

--

end CH11A_ARCH;

UCF：

##############################################

#        I/O mapping for PGA board           #

##############################################

#

NET CLK   LOC = P13;

NET XA<0> LOC = P28;

NET XA<1> LOC = P29;

NET XA<2> LOC = P35;

NET XA<3> LOC = P38;

NET XB<0> LOC = P39;

NET XB<1> LOC = P40;

NET XB<2> LOC = P44;

NET XB<3> LOC = P45;

NET XB<4> LOC = P46;

NET XB<5> LOC = P47;

NET XB<6> LOC = P48;

NET XB<7> LOC = P49;

NET YC<0> LOC = P69;

NET YC<1> LOC = P70;

NET YC<2> LOC = P77;

NET YC<3> LOC = P78;

在此我們改變

D  <= Q(22 downto 21);

使它的計時速度加快，而改變這個，更可以讓我們知道電子鐘在計時時，是什麼樣的狀態。

10.

程式：

VHDL：

entity CH06 is

port

(    CLK : in  STD_LOGIC;

YA  : out STD_LOGIC_VECTOR (3 downto 0)

);

end CH06;

architecture CH06_ARCH of CH06 is

signal Q   : STD_LOGIC_VECTOR (21 downto 0);

signal CNT : STD_LOGIC_VECTOR (1  downto 0);

begin

--

--*********************************************************

--

-- 22-BIT free counter

--

process (CLK)

begin

if CLK'event and CLK='1' then

Q <= Q + 1;

end if;

end process;

CNT <= Q(21 downto 20);

--

--*********************************************************

-- Look-Up-Table (LUT) control

--

YA <= "0001" when CNT="00" else

"0010" when CNT="01" else

"0100" when CNT="10" else

"1000" when CNT="11" else

"1111";

--

--*********************************************************

-- end of architechture

--

end CH06_ARCH;

UCF：

##############################################

#        I/O mapping for PGA board           #

##############################################

NET CLK   LOC = P13;

NET YA<0> LOC = P79;

NET YA<1> LOC = P80;

NET YA<2> LOC = P81;

NET YA<3> LOC = P82;

程式說明：

由於這次實驗，學長要求我們用第六章的程式的真值表來寫成步進馬達的程式，所以我們將UCF檔改成

NET YA<0> LOC = P79;

NET YA<1> LOC = P80;

NET YA<2> LOC = P81;

NET YA<3> LOC = P82;

使它變成馬達的接腳輸出，因為只有4種狀態，所以將計數器設成兩個BIT

signal CNT : STD_LOGIC_VECTOR (1  downto 0);

再將輸出的四種狀態變成

YA <= "0001" when CNT="00" else

"0010" when CNT="01" else

"0100" when CNT="10" else

"1000" when CNT="11" else

"1111";

使馬達輸出變成只有往上下左右在動。

11.

程式：

VHDL：

--The IEEE standard 1164 package, declares std_logic, rising_edge(), etc.

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_arith.all;

use IEEE.std_logic_unsigned.all;

--This package is a version of the Synopsys package and has been

-- optimized for use with the Express compiler.

library SYNOPSYS;

use SYNOPSYS.attributes.all;

--

--*********************************************************

-- CH15C : temperature control (test #C)

--                    designed by Pei-Chong Tang, Feb. 1999

--*********************************************************

--

entity CH15C is

port

(    CLK : in  STD_LOGIC;                      --4MHz  clock

XA  : out STD_LOGIC_VECTOR (3 downto 0);  --LED scanning

XB  : out STD_LOGIC_VECTOR (7 downto 0);  --LED segment

XC  : in  STD_LOGIC_VECTOR (3 downto 0);  --keyboard input

YA  : out STD_LOGIC_VECTOR (3 downto 0);  --PWM  output

YB  : in  STD_LOGIC_VECTOR (7 downto 0)   --photo input

);

end CH15C;

architecture CH15C_ARCH of CH15C is

component MONITOR

port

(    CLK : in    STD_LOGIC;                     --system clock

DB  : inout STD_LOGIC_VECTOR (7 downto 0); --data bus

A   : out   STD_LOGIC_VECTOR (3 downto 0); --address bus

RD  : out   STD_LOGIC;                     --read  control

SWR : out   STD_LOGIC;                     --write control

XA  : out   STD_LOGIC_VECTOR (3 downto 0); --LED scanning

XB  : out   STD_LOGIC_VECTOR (7 downto 0); --LED segment

XC  : in    STD_LOGIC_VECTOR (3 downto 0); --keyboard input

FREQ: out   STD_LOGIC                      --test frequency

);

end component;

component ADC

port

(    AOP  : out STD_LOGIC_VECTOR (7 downto 0); --ADC data

AIN  : in  STD_LOGIC_VECTOR (7 downto 0); --ADC input

STC  : out STD_LOGIC;                     --start of conv.

ENB  : out STD_LOGIC;                     --read enable

FREQ : in  STD_LOGIC;                     --sample point

CLK  : in  STD_LOGIC                      --system clock

);

end component;

signal DB   : STD_LOGIC_VECTOR (7 downto 0);

signal A    : STD_LOGIC_VECTOR (3 downto 0);

signal RD, SWR : STD_LOGIC;

signal AOP,AIN : STD_LOGIC_VECTOR (7 downto 0);

signal CMD,PWM : STD_LOGIC_VECTOR (7 downto 0);

signal CS0,PWR : STD_LOGIC;

begin

--

--*********************************************************

--

-- system connection for the experiment

--

SYSTEM_CONNECT : block

signal DIN : STD_LOGIC_VECTOR (7 downto 0);

signal FREQ,STC,ENB : STD_LOGIC;

begin

U1:  MONITOR port map (CLK, DB, A, RD, SWR, XA, XB, XC, FREQ);

U2:  ADC     port map (AOP, AIN, STC, ENB, FREQ, CLK);

YA(0) <= PWR;

YA(1) <= '1';

YA(2) <= ENB;

YA(3) <= STC;

AIN   <= YB;

CS0  <= not A(3) and not A(2) and not A(1) and not A(0);

end block SYSTEM_CONNECT;

--

--*********************************************************

-- command register

--

COMMAND_REGISTER : block

signal T,EC : STD_LOGIC;

begin

process (CLK)

begin

if CLK'event and CLK='1' then

if EC='1' then

CMD <= DB;

end if;

end if;

end process;

DB <= AOP when T='1' else "ZZZZZZZZ";

EC <= SWR and CS0;

T  <= not RD and CS0;

end block COMMAND_REGISTER;

--

--*********************************************************

-- temperature control

--

TEMP_CONTROL : block

signal DIF : STD_LOGIC_VECTOR (7 downto 0);

begin

DIF <= (CMD-AOP) when CMD>AOP else

"00000000";

PWM <= DIF;

end block TEMP_CONTROL;

--

--*********************************************************

-- PWM generator

--

PWM_GENERATOR : block

signal Q : STD_LOGIC_VECTOR (7 downto 0);

begin

process (CLK)

begin

if CLK'event and CLK='1' then  --CLK rising

Q <= Q-1;                 --down counter

end if;

end process;

PWR <= '1' when PWM>Q else '0';     --PWM output

end block PWM_GENERATOR;

--

--*********************************************************

-- end of architechture

--

end CH15C_ARCH;

UCF：

##############################################

#        I/O mapping for PGA board           #

##############################################

#

NET  CLK   LOC = P13;

NET XA<0> LOC = P28;

NET XA<1> LOC = P29;

NET XA<2> LOC = P35;

NET XA<3> LOC = P38;

NET XB<0> LOC = P39;

NET XB<1> LOC = P40;

NET XB<2> LOC = P44;

NET XB<3> LOC = P45;

NET XB<4> LOC = P46;

NET XB<5> LOC = P47;

NET XB<6> LOC = P48;

NET XB<7> LOC = P49;

NET XC<0> LOC = P50;

NET XC<1> LOC = P51;

NET XC<2> LOC = P56;

NET XC<3> LOC = P57;

NET YA<0> LOC = P79;

NET YA<1> LOC = P80;

NET YA<2> LOC = P81;

NET YA<3> LOC = P82;

NET YB<0> LOC = P59;

NET YB<1> LOC = P60;

NET YB<2> LOC = P58;

NET YB<3> LOC = P67;

NET YB<4> LOC = P61;

NET YB<5> LOC = P62;

NET YB<6> LOC = P65;

NET YB<7> LOC = P66;

另外還外加了Monitor、Adc、Pwn、Keyboard副程式。

實驗結果：

1.如果溫度設定60，最終的加熱裝置一定要能持續的做輕微加溫，才能將溫度維

持在60上。

2.但是如果設定值和量測值一致，那麼誤差直就是0;依照比例控制的公式，顯然加溫值PWM也是0。

3.既然加溫值PWM=0，顯然無法加溫，溫度當然就不能維持在60以上了。

4.所以最終的穩定值一定是: 因為一些誤差值，由比例控制而產生某個PWM值，所產生的熱量剛好可以維持一個穩定的溫度。